# CS23532 – COMPUTER NETWORKS LAB

## EXPERIMENT – 2

Study of Different Types of Network Cables

### Aim

To study different types of network cables, understand their construction, characteristics, advantages, disadvantages and applications, and learn the procedure for making Ethernet straight-through and crossover cables.

### Objectives

- Understand different types of network cables.

- Identify the applications of different cables.

- Understand straight-through and crossover cable arrangements.

- Learn the basic procedure for terminating an Ethernet cable with an RJ45 connector.

### Types of Network Cables

The following table presents the different network cables studied in this experiment. The table is divided into two parts so that the content remains clear and readable in font size 11.

| Cable Type | Construction / Type | Advantages | Common Applications |
| --- | --- | --- | --- |
| UTP | Unshielded Twisted Pair; Cat 3, Cat 5, Cat 5e, Cat 6 | Low cost, flexible, easy to install | Ethernet LANs, offices and computer networks |
| STP | Shielded Twisted Pair with protective shielding | Better protection against electromagnetic interference | Networks in electrically noisy environments |
| Coaxial Cable | Central copper conductor with insulation, shield and outer jacket | Durable and good resistance to interference | Cable TV, broadband and some communication systems |
| Fibre Optic | Uses light through optical fibre instead of electrical signals | Very high bandwidth, long distance and immune to EMI | Backbone networks, long-distance communication and high-speed links |

### Additional Ethernet Cable Types

| Cable Type | Wiring / Construction | Advantages | Common Applications |
| --- | --- | --- | --- |
| Straight-Throug h | Same wiring standard at both ends, such as T568B–T568B | Simple and widely used for normal Ethernet connections | PC–Switch, Router–Switch |

| Cable Type | Wiring / Construction | Advantages | Common Applications |
| --- | --- | --- | --- |
| Crossover | Different wiring standards at the two ends, such as T568A–T568B | Allows direct connection between compatible similar devices when required | PC–PC or direct device-to-device Ethernet connection |
| Patch Cable | Short, flexible Ethernet cable, usually factory-terminated with RJ45 connectors | Ready to use, flexible and convenient | Connecting PCs, switches, patch panels and network equipment |

### Tools and Parts Required

  - Ethernet cable (Cat5/Cat5e or Cat6)

  - RJ45 connectors – 2

  - Crimping tool

  - Cable stripper/cutter

  - Cable tester (if available)

  - RJ45 connector boots/shields (optional)

### Straight-Through Cable

A straight-through Ethernet cable has the same wiring standard at both ends. For example, T568B at both ends. It is commonly used to connect a PC to a switch or a router to a switch.

| Pin | T568B Colour |
| --- | --- |
| 1 | White/Orange |
| 2 | Orange |
| 3 | White/Green |
| 4 | Blue |
| 5 | White/Blue |
| 6 | Green |
| 7 | White/Brown |
| 8 | Brown |

### Crossover Cable

A crossover cable uses different wiring standards at the two ends. A common arrangement is T568A at one end and T568B at the other. It can be used for direct connection between similar Ethernet devices when auto-MDI/MDIX is not available.

### Procedure for Making an Ethernet Cable

- Cut the required length of Ethernet cable.

- Strip approximately 1.5 cm of the outer cable jacket from each end.

- Untwist and arrange the wires according to the required T568A or T568B colour sequence.

- Straighten the wires and trim them evenly.

- Insert the wires into the RJ45 connector in the correct order.

- Insert the connector into the crimping tool and press firmly.

- Repeat the process at the other end. Use the same standard for straight-through and different standards for crossover.

- Test the completed cable using a cable tester or by connecting suitable network devices.

### Observation

Different network cables were studied and compared based on their construction, advantages and applications. UTP, STP, coaxial and fibre optic cables were identified. Straight-through, crossover and patch cables were also studied. The procedure for arranging wires and terminating an RJ45 connector was understood.

### Result

Thus, different types of network cables were successfully studied. The differences between

straight-through and crossover Ethernet cables were understood, and the procedure for preparing and testing an RJ45 Ethernet cable was learned successfully.